/*************************************************

IoT PATIENT HEALTH & STRESS MONITORING SYSTEM

*************************************************/

// ================= LIBRARIES =================

#include <LiquidCrystal.h>

#include <SoftwareSerial.h>

#include <OneWire.h>

#include <DallasTemperature.h>

#define USE_ARDUINO_INTERRUPTS true

#include <PulseSensorPlayground.h>

// ================= LCD =================

LiquidCrystal lcd(7, 6, 5, 4, 3, 2);

// ================= ESP8266 =================

SoftwareSerial esp(10, 11);   // RX, TX

// ================= TEMPERATURE =================

#define ONE_WIRE_BUS 9

OneWire oneWire(ONE_WIRE_BUS);

DallasTemperature sensors(&oneWire);

DeviceAddress tempDeviceAddress;

// ================= GSR =================

#define GSR_PIN A1

int gsrValue = 0;

// ================= VARIABLES =================

int buzzer = 8;

float temp = 0;

const int PulseWire = A0;

int myBPM = 0;

int Threshold = 550;

PulseSensorPlayground pulseSensor;

// ================= THINGSPEAK =================

String WRITE_API_KEY = "YOUR_WRITE_API_KEY";

unsigned long previousMillis = 0;

unsigned long displayMillis  = 0;

const long interval = 15000;

const long displayInterval = 2000;

bool showGSR = false;

// ================= SETUP =================

void setup()

{

Serial.begin(9600);

esp.begin(9600);

// ===== LCD SAFE INITIALIZATION =====

lcd.begin(16, 2);

delay(200);

lcd.clear();

lcd.home();

lcd.setCursor(0,0);

lcd.print("Initializing");

delay(1500);

lcd.clear();

// Sensors

sensors.begin();

pulseSensor.analogInput(PulseWire);

pulseSensor.setThreshold(Threshold);

pulseSensor.begin();

pinMode(buzzer, OUTPUT);

digitalWrite(buzzer, LOW);

lcd.setCursor(0, 0);

lcd.print("IoT Patient");

lcd.setCursor(0, 1);

lcd.print("Monitoring");

delay(2000);

lcd.clear();

connectWiFi();

}

// ================= LOOP =================

void loop()

{

myBPM = pulseSensor.getBeatsPerMinute();

if (pulseSensor.sawStartOfBeat() && myBPM > 30)

{

digitalWrite(buzzer, HIGH);

delay(50);

digitalWrite(buzzer, LOW);

}

sensors.requestTemperatures();

if (sensors.getAddress(tempDeviceAddress, 0))

{

temp = sensors.getTempC(tempDeviceAddress);

}

gsrValue = analogRead(GSR_PIN);

unsigned long now = millis();

if (now - displayMillis >= displayInterval)

{

displayMillis = now;

showGSR = !showGSR;

}

lcd.setCursor(0, 0);

lcd.print("TEMP:");

lcd.print(temp);

lcd.print(" C   ");

lcd.setCursor(0, 1);

if (showGSR)

{

lcd.print("GSR:");

lcd.print(gsrValue);

lcd.print("      ");

}

else

{

lcd.print("HR:");

lcd.print(myBPM);

lcd.print(" BPM   ");

}

uploadThingSpeak();

}

// ================= FUNCTIONS =================

void connectWiFi()

{

esp.println("AT+RST");

delay(3000);

esp.println("AT+CWMODE=1");

delay(1000);

esp.println("AT+CWJAP=\"YOUR_WIFI_NAME\",\"YOUR_WIFI_PASSWORD\"");

delay(8000);

}

void uploadThingSpeak()

{

if (millis() - previousMillis >= interval)

{

previousMillis = millis();

esp.println("AT+CIPSTART=\"TCP\",\"api.thingspeak.com\",80");

delay(2000);

String data = "GET /update?api_key=" + WRITE_API_KEY +

"&field1=" + String(myBPM) +

"&field2=" + String(temp) +

"&field3=" + String(gsrValue) +

" HTTP/1.1\r\n"

"Host: api.thingspeak.com\r\n"

"Connection: close\r\n\r\n";

esp.print("AT+CIPSEND=");

esp.println(data.length());

delay(1500);

esp.print(data);

delay(2000);

esp.println("AT+CIPCLOSE");

}

}